RITES DPR Appraisal Tool demo

| S. No. | Check Area | Specific Check to be Tested | Required Input / Layer | Expected Output / Flag from Tool | Reference |
| --- | --- | --- | --- | --- | --- |
| 1 | Alignment |  |  |  |  |
| (a) | Alignment Data Validation | Check whether the uploaded alignment polyline matches the DPR alignment drawings / GIS file | DPR alignment file, CAD/GIS polyline, KML/Shapefile | Alignment accepted / mismatch flagged | Consultant feedback |
| (b) | Key Points to be considered during alignment |  |  |  | Consultant feedback |
|  | Defence Area/Mining area/other No-Go/restricted area | Check whether the alignment passes through these areas | API- BiSAG/GIS base map | Non-compliant alignment flagged |  |
|  | Forest/Wildlife area | Check whether the alignment passes through these areas | API-BiSAG/GIS base map Mitigation measure to be included | Non-compliant alignment flagged |  |
|  | Obligatory points | Define obligatory points like stations, economic nodes | API-BiSAG/GIS base map | Non-compliant alignment flagged |  |
|  | Yard congestion/Habitation/Land Cost | Congestion/space constraints at yards | Land Cost > 2 cr/acre | Non-Compliant flagged- bypass/ new alignment to be evaluated |  |
| (c) | Coordinate Validation | Check whether station coordinates and alignment coordinates are correctly geo-referenced | Station coordinates, GIS base map | Incorrect coordinate / misplaced station flagged |  |
| (d) | Corridor Length Check | Compare GIS-derived alignment length with DPR-reported length | DPR length statement, GIS alignment | Difference in length highlighted with percentage variation |  |
| 2 | Technical feature of alignment |  |  |  |  |
| (a) | Curve Radius Compliance | Check if curves comply with minimum radius requirements for speed of 160 km/hr and 130 kmph | 1.3 Degree (160kmph)- 2.2 Degree (130 kmph) For new line project- 1 Degree | Non-compliant curve locations flagged | Correction slip no. 10 to the Indian Railways Permanent Way Manual 2020 dated 7.11.22 |
| (b) | Gradient Compliance | Check whether vertical gradients are within permissible limits | - Ruling gradient: 1 : 100 (Compensated) - Approach to river- 1 in 150 - Station/yards-1 in 1200 or Maximum 400 | Non-compliant gradient stretches flagged |  |
| 3 | Track |  |  |  |  |
| (a) | Design Speed | Check the design speed provided in DPR | Design Speed-160 kmph | Non-Compliance to be flagged |  |
| (b) | Route Km (Rkm) | Check the RKm in line with 1 (d) | Alignment finalized | Non-Compliance to be flagged |  |
| (c) | Track Km (Tkm) | Compare the Rkm and Tkm | Tkm < 1.2 Rkm (doubling project) Tkm< 2.4 Rkm(3rd and 4th line) | Non-Compliance to be flagged |  |
| (d) | Design Axle Load | Check whether Loading standards are with permissible level | 25 tons | Non-Compliance to be flagged | Item no 3 of recommendation at page no. 67 of GE-14 |
| (e) | Minimum Formation width | Check the minimum formation width | - 7.85 m for single line - 13.15 m for double line` | Non-Compliance to be flagged |  |
| (f) | Track centre spacing | Check the minimum track centre spacing | - Minimum 5.3 m (for doubling) - Minimum 7.8 m for 3rd and 5.3 m for 4th line from 3rd line | Non-Compliance to be flagged |  |
| 4 | Bridges |  |  |  |  |
| (a) | Design loading | Loading standards | - Sub structure – 25 T loading - Super Structure – 25 T loading | Non-Compliance to be flagged |  |
| 5 | Electrification |  |  |  |  |
| (a) | Sectioning and Paralleling Post (SP) | Number of SP per Km Route Km (Rkm) | One number per 50 Km of Route Length | Non-Compliance to be flagged |  |
| (b) | Sub-Sectioning and Paralleling Post (SSP) | Number of SSP per Km Route Km (Rkm) | Two number per 50 Km of Route length | Non-Compliance to be flagged |  |
| (c) | Traction Sub Station (TSS) | Number of TSS per Km Route Km (Rkm) | One at every 50 Km of Route Length | Non-Compliance to be flagged |  |
| (d) | Tower Wagon/Car & Shed | Number of Tower Wagon/Car & Shed | One per 50 Route Km or 100 Km Track Km | Non-Compliance to be flagged |  |
| (e) | SCADA (Supervisory control and Data Acquisition) | Remote Terminal Units | Number of Remote Terminal Units > 50, then new SCADA System | Non-Compliance to be flagged |  |
| 7 | Signalling |  |  |  |  |
| (a) | Provision of EI | - Standard-I upto 50 Kmph - Standard-II upto 100 kmph - Standard-III upto 130 Kmph - Standard-IV upto 160 Kmph | Standard-IV- 160 Kmph | Non-Compliance to be flagged | RDSO |
| (b) | System of Working | - Absolute Block System - Automatic Block System - All HDN Routes - Section of HUN routes identified by Zonal Railways - All Notified sub urban section - Proposed 3rd and 4th line | Selection of Working system HDN/HUN/ Sub Urban section/ 3rd and 4th line= Automatic Signaling | Non-Compliance to be flagged |  |
| (c) | Track Circuit | HDN/GQ/GD/ Tunnel | Multi sectional Digital Axle Counters (MSDAC) | Non-Compliance to be flagged |  |
| (d) | Automatic Train Protection System | All new Projects | Kavach | Non-Compliance to be flagged |  |
| (g) | Remote Diagnostic & Predictive Maintenance System (RDPMS) | RDPMS – all Automatic route | Yes/No | Flagged off in case of “No” |  |
| 8 | Traffic |  |  |  |  |
| (a) | Line capacity | - Existing Line Capacity - with Maintenance Block - Maintenance Block with Automatic Signalling - Projected Line Capacity - With Maintenance Block - Maintenance Block with Automatic Signaling | >70% <100% | Projected Line Capacity with Maintenance Block - flagged off |  |
| (b) | Catchment Coverage | Population in Buffer Zone of 5 km, 10 km and 25 km | Population Assessment in Buffer zone using BiSAG (Yes/No) | Flagged off in case of “No” |  |
| (c) | Stakeholder Consultations | - Ministry of coal - Ministry of Steel - Ministry of Heavy Industries - Ministry of Home Affairs - Ministry of Environment, Forest & Climate Change - Ministry of Shipping, Ports and Waterways - Ministry of Tourism - Department of Telecommunication - Forest Department - District Administration - State Electricity Distribution Department - Industry Association - Steel Plants - Thermal Power Plants - Cement Plants - Fertilizer Plants - Food Corporation of India - Any other relevant Stakeholder | If (Y/N) | Flagged off in case of “No” |  |
| (d) | Tourist Destinations | List of Tourist destinations using BiSAG and existing footfalls | Yes/No | Flagged in case of “No” |  |
| (e) | Interchange Potential | Check connectivity with railway stations, metro, bus terminals, airports, major roads, Multimodal Logistic hubs | Transport network layers | Flagged in case of not undertaken |  |
| (f) | First/Last Mile connectivity to station, major economic nodes in and around project section | Assessment of existing roads/Road networks, identify requirement of widening and substantiate through photographs | Yes/No | Flagged in case of “No” |  |
| 9 | Land Requirement & Cost |  |  |  |  |
| (a) | Total Land | Bifurcation of land in Private, Government and Forest | Yes/No | Flagged off in case of “No” |  |
|  |  | State-wise land requirement | Yes/No | Flagged off in case of “No” |  |
| (b) | Land Cost | Rural factor | <2 | Flagged off in case of non-compliance |  |
|  |  | Urban factor | <1 | Flagged off in case of non-compliance |  |
|  |  | Administration Cost | >3% | Flagged off in case of “No” |  |
|  |  | Contingencies | >5% | Flagged off in case of “No” |  |
|  |  | Interest, if more than 1 year | >1 | Flagged off in case of “No” |  |
| 10 | Financial Analysis |  |  |  |  |
| (a) | FIRR | Return on Project | <10% | Flagged off |  |
| (b) | EIRR | Equity IRR | <18% | Flagged off |  |
| (c) | Total Project Cost |  |  |  |  |
|  | New Lines | Total project Cost per Rkm | >28 cr/Rkm <25 cr/Rkm | Flagged off in case of non-compliance |  |
|  | Doubling | Total Project Cost per Rkm | >18 cr/Rkm <15 cr/Rkm | Flagged off in case of non-compliance |  |
|  | Gauge Conversion | Total Project Cost per Rkm | >8 cr/Rkm < 7 cr/Rkm | Flagged off in case of non-compliance |  |