Below is a checklist table for the RITES DPR Appraisal Tool demo, specifically for testing whether an RRTS alignment is correct / optimum.

| S. No. | Check Area | Specific Check to be Tested | Required Input / Layer | Expected Output / Flag from Tool |
| --- | --- | --- | --- | --- |
| 1 | Alignment Data Validation | Check whether the uploaded alignment polyline matches the DPR alignment drawings / GIS file | DPR alignment file, CAD/GIS polyline, KML/Shapefile | Alignment accepted / mismatch flagged |
| 2 | Chainage Consistency | Verify whether station chainages, corridor length and intermediate chainages are consistent | Alignment chainage table, station list | Chainage gap / overlap / mismatch highlighted |
| 3 | Coordinate Validation | Check whether station coordinates and alignment coordinates are correctly geo-referenced | Station coordinates, GIS base map | Incorrect coordinate / misplaced station flagged |
| 4 | Corridor Length Check | Compare GIS-derived alignment length with DPR-reported length | DPR length statement, GIS alignment | Difference in length highlighted with percentage variation |
| 5 | Curve Radius Compliance | Check if horizontal curves comply with minimum radius requirements for RRTS speed | Alignment geometry, design standards | Non-compliant curve locations flagged |
| 6 | Gradient Compliance | Check whether vertical gradients are within permissible limits | Vertical profile, longitudinal section | Non-compliant gradient stretches flagged |
| 7 | Speed Feasibility | Assess whether the proposed geometry supports intended design / operating speed | Curve radius, gradient, station spacing | Speed-restricted sections identified |
| 8 | Station Spacing | Check whether station-to-station distance is reasonable for RRTS operations | Station list, chainages | Very close / very far station spacing flagged |
| 9 | Station Location Logic | Check whether stations are located near major demand nodes | Population, employment, land use, activity centres | Weak / strong catchment classification |
| 10 | Catchment Coverage | Assess 500 m, 1 km, 2 km or suitable catchment around each station | GIS land use, population, road network | Station-wise catchment score generated |
| 11 | Interchange Potential | Check connectivity with railway stations, metro, bus terminals, ISBTs, airports, major roads | Transport network layers | Missing / strong interchange opportunities identified |
| 12 | First / Last Mile Access | Assess road access, feeder bus potential, parking, NMT access around stations | Road network, existing transport nodes | Poor access locations flagged |
| 13 | Existing ROW Utilisation | Check how much alignment follows existing transport corridors / available ROW | Road ROW, railway ROW, government land | Percentage of alignment within available ROW |
| 14 | Land Acquisition Impact | Identify private land, built-up parcels, institutional land and sensitive parcels affected | Cadastral / parcel / land use data | High land acquisition zones flagged |
| 15 | Built-up Impact | Check whether alignment passes through dense built-up areas | Built-up layer, satellite imagery | High demolition / resettlement risk flagged |
| 16 | Constructability Check | Assess whether alignment is feasible based on road width, built-up density and site constraints | Road width, land use, structure type | Difficult construction stretches identified |
| 17 | Structural Typology Validation | Check appropriateness of elevated, at-grade, underground, bridge or special span sections | Alignment profile, road width, crossings | Typology mismatch flagged |
| 18 | Major Road Crossings | Identify NH, SH, arterial roads and junction crossings along alignment | Road network layer | Crossing inventory generated |
| 19 | Railway Crossings | Identify railway line crossings and required ROB / underpass / special structures | Railway network layer | Railway conflict locations flagged |
| 20 | Water Body / Drain Crossings | Identify rivers, canals, drains, lakes and flood-prone crossings | Hydrology layer, flood layer | Bridge / culvert / special span requirement flagged |
| 21 | Utility Conflict | Identify major utilities such as transmission lines, pipelines, substations, drains, etc. | Utility GIS layers | Utility shifting risk locations flagged |
| 22 | Environmental Sensitivity | Check overlap with forests, protected areas, water bodies, green zones and eco-sensitive areas | Environmental layers | Sensitive impact zones flagged |
| 23 | Social Sensitivity | Check proximity to schools, hospitals, religious places, heritage areas and dense settlements | POI / social infrastructure layers | Sensitive receptors identified |
| 24 | Depot Connectivity | Check whether depot location is operationally connected with the main alignment | Depot location, alignment, track connection | Depot access issue flagged |
| 25 | Terminal / Turnback Feasibility | Check whether terminal stations have adequate operational space and turnback arrangement | Station layout, track layout | Terminal constraint flagged |
| 26 | Future Extension Potential | Check whether alignment allows logical future extension | Regional plan, future corridor plans | Extension feasibility score generated |
| 27 | Demand Coverage | Check whether alignment connects high-demand origin-destination corridors | OD demand, population, employment, trip data | Corridor demand coverage score generated |
| 28 | Travel Time Benefit | Compare expected travel time with road / existing public transport alternatives | Speed assumptions, road travel time | Travel time saving estimated |
| 29 | Cost Sensitivity | Identify sections likely to increase cost due to land, structures, utilities or environment | Cost norms, risk layers | High-cost sections flagged |
| 30 | Alternative Alignment Comparison | Compare proposed alignment with one or more alternate options | Alternative GIS alignments | Comparative scorecard generated |
| 31 | Risk Register Generation | Prepare location-wise risk register for alignment issues | All GIS and DPR inputs | Risk register with chainage-wise issues |
| 32 | Overall Alignment Score | Generate final alignment suitability / optimisation score | All technical, GIS, demand and risk parameters | Alignment classified as suitable / needs review / high risk |